EVERCLEAR

Non-Yellowing Acrylic Cure and Seal for Decorative Concrete

EVERCLEAR is a pure acrylic concrete cure & seal that protects and enhances the appearance of cured concrete with a clear, non-yellowing seal that is harder and more durable than standard cure and seal products.

{Note to Specifier: The paragraphs below are meant to be incorporated into Parts 2 and 3 of a standard CSI 3 Part Format specification, project’s General Structural Notes or directly onto the plans. They must be carefully reviewed by a qualified design professional and edited to meet the particular requirements of the project at hand, assure compliance with any governing building codes, and coordinate with other specification sections and drawings.}

PART 2:	PRODUCT

2.__CURING AND SEALING COMPOUND

A.	Concrete curing and sealing compound. A ready-to-use, acrylic curing and sealing compound for use on new and existing, interior and exterior, horizontal and vertical concrete surfaces. Product shall conform to:

ASTM C 309, Type 1 Class A & B and ASTM C 1315 Type 1 Class A.

1.	 Product:

- Euclid Chemical Company (The); EverClear www.euclidchemical.com

2.__	Manufacturer shall have ISO 9001 Quality Certification.

PART 3: 	EXECUTION

3.__ 	EXAMINATION

A. 	When used on existing concrete, examine concrete surfaces to receive curing and sealing compound. Notify specifier if surfaces are not acceptable. Do not begin surface preparation or application until unacceptable conditions are corrected.

3.__	 SURFACE PREPARATION

A. 	When used on existing concrete, prepare concrete surfaces in accordance with manufacturer’s written instructions.

B. 	Ensure surfaces are clean and free of standing water.

C. 	Remove dirt, dust, oil, grease, sealers, and other materials that may prevent adhesion of curing and sealing compound.

3.__ 	APPLICATION

A.	For the best cure of freshly placed concrete, apply EVERCLEAR as soon as possible after finishing operations and/or immediately after the disappearance of the “sheen” of surface moisture.

B. 	Apply curing and sealing compound to concrete surfaces in accordance with manufacturer’s written instructions.

C. 	Apply compound at uniform coverage rate in accordance with manufacturer’s written instructions.

D. 	Apply as soon as possible after finishing and immediately after disappearance of surface moisture sheen, when used to cure freshly placed concrete.

E. 	Do not apply to concrete to receive toppings, epoxy coatings, urethane coatings, or epoxy adhesives.

F. 	Do not use as a bond breaker for tilt-up construction.

G. 	Do not dilute curing and sealing compound.

H. 	Cure concrete surfaces as specified in Section 03300.